PIX with Dynamic DSL Connection to Astaro ASG425 running 7.304

sysopt connection permit-ipsec

crypto ipsec transform-set AMLI_3DES_DH1 esp-3des esp-md5-hmac

crypto map mymap 50 ipsec-isakmp

crypto map mymap 50 match address 101

crypto map mymap 50 set peer 38.104.xx.xx

crypto map mymap 50 set transform-set AMLI_3DES_DH1

crypto map mymap client configuration address initiate

crypto map mymap interface outside

isakmp enable outside

isakmp key ******** address 38.104.xx.xx netmask 255.255.255.255

isakmp identity address

isakmp keepalive 90

isakmp policy 50 authentication pre-share

isakmp policy 50 encryption 3des

isakmp policy 50 hash md5

isakmp policy 50 group 1

isakmp policy 50 lifetime 86400

ASG Site-to-Site settings:

IPSEC POLICY:

REMOTE GATEWAY:

VPN TUNNEL CONFIGURATION: